# Core Switch DHCP + VRRP Configuration Documentation

This document provides the detailed configuration for Core-SW1 and Core-SW2. It includes VRRP setup for gateway redundancy and integrated DHCP server configuration divided equally between the two core switches. Each VLAN uses a /24 subnet with .1 as the default gateway (via VRRP) and DHCP pools distributed.

# VLANs and IP Schema

| VLAN ID | Subnet | Default Gateway | Purpose |
| --- | --- | --- | --- |
| 50 | 10.0.5.0/24 | 10.0.5.1 | Core business applications (ERP, finance) |
| 60 | 10.0.6.0/24 | 10.0.6.1 | HR systems / restricted systems |
| 70 | 10.0.7.0/24 | 10.0.7.1 | IT engineers / admin workstations |
| 80 | 10.0.8.0/24 | 10.0.8.1 | IP Phones (native voice VLAN) |
| 90 | 10.0.9.0/24 | 10.0.9.1 | IP cameras / video storage |
| 100 | 10.0.10.0/24 | 10.0.10.1 | Guest WiFi / internet-only clients |
| 110 | 10.0.11.0/24 | 10.0.11.1 | Dev/QA lab / sensitive environments |
| 200 | 10.0.20.0/24 | 10.0.20.1 | Printers, IoT sensors, building automation |

# Core-SW1 Configuration

system-view
#
# Enable DHCP globally
dhcp enable
#
# VLAN Interfaces with VRRP
#
interface Vlanif50
 ip address 10.0.5.2 255.255.255.0
 vrrp vrid 50 virtual-ip 10.0.5.1
 vrrp vrid 50 priority 120
#
interface Vlanif60
 ip address 10.0.6.2 255.255.255.0
 vrrp vrid 60 virtual-ip 10.0.6.1
 vrrp vrid 60 priority 120
#
interface Vlanif70
 ip address 10.0.7.2 255.255.255.0
 vrrp vrid 70 virtual-ip 10.0.7.1
 vrrp vrid 70 priority 120
#
interface Vlanif80
 ip address 10.0.8.2 255.255.255.0
 vrrp vrid 80 virtual-ip 10.0.8.1
 vrrp vrid 80 priority 120
#
interface Vlanif90
 ip address 10.0.9.2 255.255.255.0
 vrrp vrid 90 virtual-ip 10.0.9.1
 vrrp vrid 90 priority 120
#
interface Vlanif100
 ip address 10.0.10.2 255.255.255.0
 vrrp vrid 100 virtual-ip 10.0.10.1
 vrrp vrid 100 priority 120
#
interface Vlanif110
 ip address 10.0.11.2 255.255.255.0
 vrrp vrid 110 virtual-ip 10.0.11.1
 vrrp vrid 110 priority 120
#
interface Vlanif200
 ip address 10.0.20.2 255.255.255.0
 vrrp vrid 200 virtual-ip 10.0.20.1
 vrrp vrid 200 priority 120
#
# DHCP Server - Core-SW1 handles VLANs 50, 70, 90, 110
#
ip pool VLAN50-POOL
 gateway-list 10.0.5.1
 network 10.0.5.0 mask 255.255.255.0
 excluded-ip-address 10.0.5.1 10.0.5.9
 dns-list 8.8.8.8
#
ip pool VLAN70-POOL
 gateway-list 10.0.7.1
 network 10.0.7.0 mask 255.255.255.0
 excluded-ip-address 10.0.7.1 10.0.7.9
 dns-list 8.8.8.8
#
ip pool VLAN90-POOL
 gateway-list 10.0.9.1
 network 10.0.9.0 mask 255.255.255.0
 excluded-ip-address 10.0.9.1 10.0.9.9
 dns-list 8.8.8.8
#
ip pool VLAN110-POOL
 gateway-list 10.0.11.1
 network 10.0.11.0 mask 255.255.255.0
 excluded-ip-address 10.0.11.1 10.0.11.9
 dns-list 8.8.8.8
#
return

# Core-SW2 Configuration

system-view
#
# Enable DHCP globally
dhcp enable
#
# VLAN Interfaces with VRRP
#
interface Vlanif50
 ip address 10.0.5.3 255.255.255.0
 vrrp vrid 50 virtual-ip 10.0.5.1
 vrrp vrid 50 priority 110
#
interface Vlanif60
 ip address 10.0.6.3 255.255.255.0
 vrrp vrid 60 virtual-ip 10.0.6.1
 vrrp vrid 60 priority 110
#
interface Vlanif70
 ip address 10.0.7.3 255.255.255.0
 vrrp vrid 70 virtual-ip 10.0.7.1
 vrrp vrid 70 priority 110
#
interface Vlanif80
 ip address 10.0.8.3 255.255.255.0
 vrrp vrid 80 virtual-ip 10.0.8.1
 vrrp vrid 80 priority 110
#
interface Vlanif90
 ip address 10.0.9.3 255.255.255.0
 vrrp vrid 90 virtual-ip 10.0.9.1
 vrrp vrid 90 priority 110
#
interface Vlanif100
 ip address 10.0.10.3 255.255.255.0
 vrrp vrid 100 virtual-ip 10.0.10.1
 vrrp vrid 100 priority 110
#
interface Vlanif110
 ip address 10.0.11.3 255.255.255.0
 vrrp vrid 110 virtual-ip 10.0.11.1
 vrrp vrid 110 priority 110
#
interface Vlanif200
 ip address 10.0.20.3 255.255.255.0
 vrrp vrid 200 virtual-ip 10.0.20.1
 vrrp vrid 200 priority 110
#
# DHCP Server - Core-SW2 handles VLANs 60, 80, 100, 200
#
ip pool VLAN60-POOL
 gateway-list 10.0.6.1
 network 10.0.6.0 mask 255.255.255.0
 excluded-ip-address 10.0.6.1 10.0.6.9
 dns-list 8.8.8.8
#
ip pool VLAN80-POOL
 gateway-list 10.0.8.1
 network 10.0.8.0 mask 255.255.255.0
 excluded-ip-address 10.0.8.1 10.0.8.9
 dns-list 8.8.8.8
#
ip pool VLAN100-POOL
 gateway-list 10.0.10.1
 network 10.0.10.0 mask 255.255.255.0
 excluded-ip-address 10.0.10.1 10.0.10.9
 dns-list 8.8.8.8
#
ip pool VLAN200-POOL
 gateway-list 10.0.20.1
 network 10.0.20.0 mask 255.255.255.0
 excluded-ip-address 10.0.20.1 10.0.20.9
 dns-list 8.8.8.8
#
return