Enterprise Network Design and Implementation Using Huawei Datacom Technologies

Core Link Technologies

Full Documentation

## Core Link Technologies – Full Documentation

## 1. Executive Summary

This document provides a complete technical design and configuration reference for Core Link Technologies network including a Headquarters (HQ) and Branch (BR1), interconnected through ISP routers using public IPs, static routing, RIP, and a GRE tunnel.

The design includes a full three‑tier switching architecture (Core, Aggregation, Access), VRRP high availability, LACP EtherChannel redundancy, wireless infrastructure with ACs and AP groups, DHCP services, AAA security, and IP addressing schemas for all VLANs.

## 2. Network Overview

The network includes two main sites: Headquarters (HQ) and Branch (BR1).

Connectivity between sites is achieved via ISP routers and a GRE tunnel carried over public IP addressing.

HQ uses dual core switches with full VRRP gateway redundancy and LACP-based inter-switch uplinks.

Each site features VLAN segmentation for HR, IT, CCTV, VOIP, Guest, Servers, Management, IoT, and Backup.

Wireless services are centrally managed through dual ACs (AC-HQ and AC-Suez) and CAPWAP AP deployment.

## 3. Physical & Interface Mapping

A mapping of all inter-device connections including router-to-router, core-to-aggregation, aggregation-to-access, and access-to-AP infrastructure.

All EtherChannel (Eth-Trunk) bundles are documented, including port assignments and uplink distribution.

🏢 HQ Site

## HQ Core

| Device | Interface | Connected To | Interface | Notes |
| --- | --- | --- | --- | --- |
| HQ-R1 | G0/0/0 | HQ-1 | G0/0/1 |  |
| HQ-R1 | G0/0/1 | ISP1 | G0/0/0 |  |
| HQ-1 | G0/0/24 | Core-SW1 | G0/0/22 |  |
| HQ-1 | G0/0/23 | Core-SW2 | G0/0/22 |  |
| Core-SW1 | G0/0/22 | HQ-1 | G0/0/24 |  |
| Core-SW1 | G0/0/23,24 | Core-SW2 | G0/0/23,24 | EtherChannel |
| Core-SW1 | G0/0/19,20 | Agg-SW1 | G0/0/23,24 | EtherChannel |
| Core-SW1 | G0/0/17,18 | Agg-SW2 | G0/0/23,24 | EtherChannel |
| Core-SW1 | G0/0/15,16 | Agg-SW3 | G0/0/23,24 | EtherChannel |
| Core-SW2 | G0/0/23,24 | Core-SW1 | G0/0/23,24 | EtherChannel |
| Core-SW2 | G0/0/19,20 | Agg-SW3 | G0/0/21,22 | EtherChannel |
| Core-SW2 | G0/0/17,18 | Agg-SW2 | G0/0/21,22 | EtherChannel |
| Core-SW2 | G0/0/15,16 | Agg-SW1 | G0/0/21,22 | EtherChannel |

### HQ Aggregation & Access

| Device | Interface | Connected To | Interface | Notes |
| --- | --- | --- | --- | --- |
| Agg-SW1 | G0/0/23,24 | Core-SW1 | G0/0/19,20 | EtherChannel |
| Agg-SW1 | G0/0/21,22 | Core-SW2 | G0/0/15,16 | EtherChannel |
| Agg-SW2 | G0/0/23,24 | Core-SW1 | G0/0/17,18 | EtherChannel |
| Agg-SW2 | G0/0/21,22 | Core-SW2 | G0/0/17,18 | EtherChannel |
| Agg-SW3 | G0/0/23,24 | Core-SW1 | G0/0/15,16 | EtherChannel |
| Agg-SW3 | G0/0/21,22 | Core-SW2 | G0/0/19,20 | EtherChannel |
| Agg-SW3 | G0/0/19,20 | AC1 | G0/0/7,8 | EtherChannel |
| Agg-SW3 | G0/0/3 | VoIP Server | Ethernet0/0/0 |  |
| Agg-SW3 | G0/0/2 | NVR Server | Ethernet0/0/0 |  |
| Agg-SW3 | G0/0/1 | File Storage | Ethernet0/0/0 |  |
| AC1 | G0/0/7,8 | Agg-SW3 | G0/0/19,20 | EtherChannel |
| AC1 | G0/0/2 | AP3 | G0/0/4 |  |
| AC1 | G0/0/4 | AP1 | G0/0/4 |  |
| AC1 | G0/0/6 | AP2 | G0/0/4 |  |

## 🏬 Branch Site (BR1)

| Device | Interface | Connected To | Interface | Notes |
| --- | --- | --- | --- | --- |
| BR1-R1 | G0/0/1 | ISP2 | G0/0/1 |  |
| BR1-R1 | G0/0/0 | BR-1 | G0/0/1 |  |
| BR-1 | G0/0/23 | BR1-CoreSwitch1 | G0/0/16 |  |
| BR-1 | G0/0/24 | BR1-CoreSwitch2 | G0/0/16 |  |
| CoreSwitch1 | G0/0/21,22 | CoreSwitch2 | G0/0/21,22 | EtherChannel |
| CoreSwitch1 | G0/0/17,18 | AG-SW-1 | G0/0/23,24 | EtherChannel |
| CoreSwitch1 | G0/0/19,20 | AG-SW-2 | G0/0/21,22 | EtherChannel |
| CoreSwitch2 | G0/0/21,22 | CoreSwitch1 | G0/0/21,22 | EtherChannel |
| CoreSwitch2 | G0/0/17,18 | AG-SW-1 | G0/0/21,22 | EtherChannel |
| CoreSwitch2 | G0/0/19,20 | AG-SW-2 | G0/0/21,22 | EtherChannel |
| AG-SW-1 | G0/0/23,24 | CoreSwitch1 | G0/0/17,18 | EtherChannel |
| AG-SW-1 | G0/0/21,22 | CoreSwitch2 | G0/0/17,18 | EtherChannel |
| AG-SW-2 | G0/0/23,24 | CoreSwitch2 | G0/0/19,20 | EtherChannel |
| AG-SW-2 | G0/0/21,22 | CoreSwitch1 | G0/0/20,19 | EtherChannel |
| AG-SW-2 | G0/0/16 | AC2 | G0/0/8 |  |
| AG-SW-2 | G0/0/17 | File Storage 2 | Ethernet0/0/0 |  |
| AG-SW-2 | G0/0/18 | NVR Server 2 | Ethernet0/0/0 |  |
| AG-SW-2 | G0/0/19 | VoIP Server 2 | Ethernet0/0/0 |  |
| AC2 | G0/0/4 | AP7 | G0/0/0 |  |
| AC2 | G0/0/6 | AP6 | G0/0/0 |  |

### ISP Chain

| Device | Interface | Connected To | Interface |
| --- | --- | --- | --- |
| ISP1 | G0/0/1 | HQ-R1 | G0/0/1 |
| ISP1 | G0/0/0 | ISP2 | G0/0/0 |
| ISP2 | G0/0/0 | ISP1 | G0/0/0 |
| ISP2 | G0/0/1 | BR1-R1 | G0/0/1 |

## 4. VLAN and IP Addressing Plan

VLANs include Management, IT, HR, CCTV, Guest, VOIP, Core Applications, Servers, DMZ, IoT, Backup, and WLAN SSIDs.

Each VLAN is assigned a subnet and VRRP virtual gateway on the core switches for redundancy.

DHCP pools are configured on Core-SW1 and Core-SW2 (HQ) and similarly on BR1 cores.
Here’s your HQ VLAN configuration arranged in a clean, structured table:

### 🏢 HQ VLAN Configuration Table

| VLAN ID | Subnet | Default Gateway | Purpose |
| --- | --- | --- | --- |
| 10 | 10.0.0.0/24 | 10.0.0.1 | Network device management (switches, routers) |
| 20 | 10.0.1.0/24 | 10.0.1.1 | Network infra (NTP, DNS internal, AD DS replicas) |
| 30 | 10.0.2.0/23 | 10.0.2.1 | Application servers, DB, internal services (HQ primary) |
| 40 | 10.0.4.0/24 | 10.0.4.1 | Public-facing services (webmail, reverse-proxy, APIs) |
| 50 | 10.0.5.0/24 | 10.0.5.1 | Core business applications (ERP, finance) |
| 60 | 10.0.6.0/24 | 10.0.6.1 | HR systems / restricted systems |
| 70 | 10.0.7.0/24 | 10.0.7.1 | IT engineers / admin workstations |
| 80 | 10.0.8.0/24 | 10.0.8.1 | IP Phones (native voice VLAN) |
| 90 | 10.0.9.0/24 | 10.0.9.1 | IP cameras / video storage |
| 100 | 10.0.10.0/24 | 10.0.10.1 | Guest WIFI / internet-only clients |
| 110 | 10.0.11.0/24 | 10.0.11.1 | Dev/QA lab / sensitive environments |
| 120 | 10.0.12.0/24 | 10.0.12.1 | AP management and controller traffic |
| 200 | 10.0.20.0/24 | 10.0.20.1 | Printers, IoT sensors, building automation |
| 250 | 10.0.21.0/24 | 10.0.21.1 | Backup network / replication |

Here’s the VLAN configuration for 🏬 Branch 1 arranged in a structured table:

### 🏬 Branch 1 VLAN Configuration Table

| VLAN ID | Subnet | Default Gateway | Purpose |
| --- | --- | --- | --- |
| 10 | 10.2.0.0/24 | 10.2.0.1 | Network device management (switches, routers) |
| 20 | 10.2.1.0/24 | 10.2.1.1 | Network infra (NTP, DNS internal, AD DS replicas) |
| 30 | 10.2.2.0/23 | 10.2.2.1 | Application servers, DB, internal services (branch servers) |
| 40 | 10.2.4.0/24 | 10.2.4.1 | Public-facing services (branch proxy, local apps) |
| 50 | 10.2.5.0/24 | 10.2.5.1 | Core business applications (ERP access, finance) |
| 60 | 10.2.6.0/24 | 10.2.6.1 | HR systems / restricted systems |
| 70 | 10.2.7.0/24 | 10.2.7.1 | IT engineers / admin workstations |
| 80 | 10.2.8.0/24 | 10.2.8.1 | IP Phones (native voice VLAN) |
| 90 | 10.2.9.0/24 | 10.2.9.1 | IP cameras / video storage |
| 100 | 10.2.10.0/24 | 10.2.10.1 | Guest WIFI / internet-only clients |
| 110 | 10.2.11.0/24 | 10.2.11.1 | Dev/QA lab / sensitive environments |
| 120 | 10.2.12.0/24 | 10.2.12.1 | AP management and controller traffic |
| 200 | 10.2.20.0/24 | 10.2.20.1 | Printers, IoT sensors, building automation |
| 250 | 10.2.21.0/24 | 10.2.21.1 | Backup network / replication |

## 5. Routing Design

HQ and Branch are connected via a GRE tunnel established between BR1-R1 and HQ router.

Static routing ensures default route propagation to ISP next-hop.

RIP version 2 is used internally between routers to advertise internal networks through the GRE tunnel.

Routing table convergence allows full reachability between HQ subnets and BR1 subnets.

## 6. Redundancy & High Availability

Dual Core switches at HQ and BR1 use VRRP for gateway redundancy.

LACP EtherChannels provide link-level redundancy between core, aggregation, and access layers.

RSTP ensures loop-free topology with fast convergence.

Wireless APs are dual-registered across AC-HQ and AC-Suez for failover.

## 7. Wireless Network Design

Wireless network uses Huawei ACs for centralized management.

SSID 'Corelink Technologies' is broadcast using WPA2-PSK security.

APs are grouped under CORELINK-APs group with both radios broadcasting same SSID.

CAPWAP tunnels connect APs to AC-HQ or AC-Suez depending on site.

## 8. Security Architecture

AAA used for admin authentication on network devices.

SSH secure algorithms set (AES, SHA256).

Illegal MAC address alarms enabled across all switches.

STP edge-port disabled on trunks to prevent rogue STP influence.

DHCP exclusion ranges used to protect infrastructure addresses.

## 9. Testing & Validation Procedures

Connectivity Tests:

• Ping between HQ and Branch via GRE tunnel.

• Ping between VLANs inside HQ and BR1.

Pc 1 from HQ to FTP server in BR1

• DHCP address assignment validation in all VLANs.

Failover Tests:

• VRRP Master/Backup role switching.

Wireless Tests:

• AP registration on AC.

• SSID visibility and client connectivity.

## 10. Troubleshooting Guide

Common Issues and Fixes:

• GRE Tunnel Down: Check source/destination public IPs and route to remote endpoint.

• VRRP Not Master: Verify priority settings and interface status.

• LACP Mismatch: Ensure trunk ports have identical configuration on both ends.

• DHCP Not Assigning: Confirm interface VLANIF DHCP select global/relay.

• AP Not Joining: Check CAPWAP source IP and routing to AC.

## 11. Maintenance Procedures

• Regular backup of switch and router configurations.

• Monitor LLDP/NDP/NTDP for topology changes.

• Verify VRRP state regularly for high availability.

• Apply firmware updates during maintenance windows.

## 12. Detailed Configuration Documentation

- HQ main router

sysname HQ-R1

ftp server enable

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

clock timezone China-Standard-Time minus 08:00:00

portal local-server load flash:/portalpage.zip

drop illegal-mac alarmundo info-center enable

lldp enable

wlan ac-global carrier id other ac id 0

set cpu-usage threshold 80 restore 75

acl number 3000

rule 5 permit ip source 10.0.10.0 0.0.0.255

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user depi password cipher %$%${j,`W<&8x=V}=<4Lz4(/8X_S%$%$

local-user depi privilege level 10

local-user depi service-type telnet ssh ftp

local-user admin password cipher %$%$PO%0CTa%]-BUo8)@.rq%8X_1%$%$

local-user admin privilege level 15

local-user admin service-type telnet ssh ftp

local-user viewer password cipher %$%$qgRW:E5B<blJQ!fMNJ58X_L%$%$

local-user viewer privilege level 0

local-user viewer service-type telnet ssh ftp

firewall zone Local

priority 15

interface GigabitEthernet0/0/0

ip address 10.0.0.10 255.255.255.0

interface GigabitEthernet0/0/1

ip address 1.1.1.2 255.255.255.0

nat outbound 3000

interface GigabitEthernet0/0/2

interface NULL0

interface Tunnel0/0/0

ip address 172.16.100.1 255.255.255.252

tunnel-protocol gre

source GigabitEthernet0/0/1

destination 1.2.1.2

stelnet server enable

ip route-static 0.0.0.0 0.0.0.0 1.1.1.1

ip route-static 1.2.1.2 255.255.255.255 1.1.1.1

ip route-static 10.2.0.0 255.255.0.0 172.16.100.2

user-interface con 0

authentication-mode password

user-interface vty 0 4

authentication-mode aaa

idle-timeout 10 0

user-interface vty 16 20

wlan ac

return

<HQ-R1>

- HQ Core-sw-1

sysname Core-SW1

FTP server enable

undo info-center enable

vlan batch 10 20 30 40 50 60 70 80 90 100

vlan batch 110 120 200 250

stp mode rstp

stp instance 0 priority 0

cluster enable

ntdp enable

ndp enable

lldp enable

undo nap slave enable

drop illegal-mac alarm

dhcp enable

diffserv domain default

acl number 3999

rule 5 permit ip source 0.0.0.0 0 destination 255.255.255.255 0

rule 10 permit ip source 10.0.0.0 0.0.0.255 destination 10.0.0.0 0.0.0.255

rule 15 permit ip source 10.0.1.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 20 permit ip source 10.0.2.0 0.0.1.255 destination 10.0.2.0 0.0.1.255

rule 25 permit ip source 10.0.4.0 0.0.0.255 destination 10.0.4.0 0.0.0.255

rule 30 permit ip source 10.0.5.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 35 permit ip source 10.0.6.0 0.0.0.255 destination 10.0.6.0 0.0.0.255

rule 40 permit ip source 10.0.7.0 0.0.0.255 destination 10.0.7.0 0.0.0.255

rule 45 permit ip source 10.0.8.0 0.0.0.255 destination 10.0.8.0 0.0.0.255

rule 50 permit ip source 10.0.9.0 0.0.0.255 destination 10.0.9.0 0.0.0.255

rule 55 permit ip source 10.0.10.0 0.0.0.255 destination 10.0.10.0 0.0.0.255

rule 60 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.11.0 0.0.0.255

rule 65 permit ip source 10.0.12.0 0.0.0.255 destination 10.0.12.0 0.0.0.255

rule 70 permit ip source 10.0.20.0 0.0.0.255 destination 10.0.20.0 0.0.0.255

rule 75 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 80 permit ip source 10.0.0.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 85 permit ip source 10.0.0.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 90 permit ip source 10.0.0.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 95 permit ip source 10.0.2.0 0.0.1.255 destination 10.0.1.0 0.0.0.255

rule 100 permit ip source 10.0.2.0 0.0.1.255 destination 10.0.5.0 0.0.0.255

rule 105 permit ip source 10.0.2.0 0.0.1.255 destination 10.0.21.0 0.0.0.255

rule 110 permit ip source 10.0.4.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 115 permit ip source 10.0.4.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 120 permit ip source 10.0.5.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 125 permit ip source 10.0.5.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 130 permit ip source 10.0.5.0 0.0.0.255 destination 10.0.6.0 0.0.0.255

rule 135 permit ip source 10.0.6.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 140 permit ip source 10.0.6.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 145 permit ip source 10.0.7.0 0.0.0.255 destination 10.0.0.0 0.0.15.255

rule 150 deny ip source 10.0.7.0 0.0.0.255 destination 10.0.10.0 0.0.0.255

rule 155 deny ip source 10.0.7.0 0.0.0.255 destination 10.0.20.0 0.0.0.255

rule 160 deny ip source 10.0.7.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 165 permit ip source 10.0.8.0 0.0.0.255 destination 10.0.12.0 0.0.0.255

rule 170 permit ip source 10.0.9.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 175 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 180 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 185 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 190 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 195 permit ip source 10.0.12.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 200 permit ip source 10.0.20.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 205 permit ip source 10.0.20.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 210 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 215 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 220 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.6.0 0.0.0.255

rule 225 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.9.0 0.0.0.255

rule 230 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.11.0 0.0.0.255

rule 235 deny ip source 10.0.10.0 0.0.0.255 destination 10.0.0.0 0.0.255.255

rule 240 deny ip source 10.0.20.0 0.0.0.255 destination 10.0.0.0 0.0.255.255

rule 245 permit ip source 10.0.1.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 250 permit ip source 10.0.2.0 0.0.1.255 destination 10.2.2.0 0.0.1.255

rule 255 permit ip source 10.0.5.0 0.0.0.255 destination 10.2.6.0 0.0.0.255

rule 260 permit ip source 10.0.11.0 0.0.0.255 destination 10.2.11.0 0.0.0.255

rule 265 permit ip source 10.0.21.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 270 deny ip

drop-profile default

ip pool vlan100-sw1

gateway-list 10.0.10.1

network 10.0.10.0 mask 255.255.255.0

excluded-ip-address 10.0.10.127 10.0.10.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan100-sw1

gateway-list 10.0.10.1

network 10.0.10.0 mask 255.255.255.0

excluded-ip-address 10.0.10.127 10.0.10.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan110-sw1

gateway-list 10.0.11.1

network 10.0.11.0 mask 255.255.255.0

excluded-ip-address 10.0.11.127 10.0.11.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan200-sw1

gateway-list 10.0.20.1

network 10.0.20.0 mask 255.255.255.0

excluded-ip-address 10.0.20.127 10.0.20.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan50-sw1

gateway-list 10.0.5.1

network 10.0.5.0 mask 255.255.255.0

excluded-ip-address 10.0.5.127 10.0.5.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan60-sw1

gateway-list 10.0.6.1

network 10.0.6.0 mask 255.255.255.0

excluded-ip-address 10.0.6.127 10.0.6.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan70-sw1

gateway-list 10.0.7.1

network 10.0.7.0 mask 255.255.255.0

excluded-ip-address 10.0.7.127 10.0.7.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan80-sw1

gateway-list 10.0.8.1

network 10.0.8.0 mask 255.255.255.0

excluded-ip-address 10.0.8.127 10.0.8.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan90-sw1

gateway-list 10.0.9.1

network 10.0.9.0 mask 255.255.255.0

excluded-ip-address 10.0.9.127 10.0.9.254

dns-list 8.8.8.8 1.1.1.1

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user depi password cipher T7O"M%@"'AC=K6%QE'XW;1!!

local-user depi privilege level 10

local-user depi service-type telnet ssh ftp

local-user admin password cipher !<*PMY9[$U)NZPO3JBXBHA!!

local-user admin privilege level 15

local-user admin service-type telnet ssh ftp

local-user viewer password cipher O.]B_,=PL+/J*EP&JX-W=1!!

local-user viewer privilege level 0

local-user viewer service-type telnet ssh ftp

interface Vlanif1

interface Vlanif10

description MGMT

ip address 10.0.0.2 255.255.255.0

vrrp vrid 10 virtual-ip 10.0.0.1

vrrp vrid 10 priority 120

dhcp select global

interface Vlanif20

description INFRA

ip address 10.0.1.2 255.255.255.0

vrrp vrid 20 virtual-ip 10.0.1.1

vrrp vrid 20 priority 120

dhcp select global

interface Vlanif30

description SERVERS

ip address 10.0.2.2 255.255.254.0

vrrp vrid 30 virtual-ip 10.0.2.1

vrrp vrid 30 priority 120

dhcp select global

interface Vlanif40

description DMZ

ip address 10.0.4.2 255.255.255.0

vrrp vrid 40 virtual-ip 10.0.4.1

vrrp vrid 40 priority 120

dhcp select global

interface Vlanif50

description CORE_APPS

ip address 10.0.5.2 255.255.255.0

vrrp vrid 50 virtual-ip 10.0.5.1

vrrp vrid 50 priority 120

dhcp select global

interface Vlanif60

description HR

ip address 10.0.6.2 255.255.255.0

vrrp vrid 60 virtual-ip 10.0.6.1

vrrp vrid 60 priority 120

dhcp select global

interface Vlanif70

description IT

ip address 10.0.7.2 255.255.255.0

vrrp vrid 70 virtual-ip 10.0.7.1

vrrp vrid 70 priority 120

dhcp select global

dhcp select relay

interface Vlanif80

description VOICE

ip address 10.0.8.2 255.255.255.0

vrrp vrid 80 virtual-ip 10.0.8.1

vrrp vrid 80 priority 120

dhcp select global

interface Vlanif90

description CCTV

ip address 10.0.9.2 255.255.255.0

vrrp vrid 90 virtual-ip 10.0.9.1

vrrp vrid 90 priority 120

dhcp select global

interface Vlanif100

description GUEST

ip address 10.0.10.2 255.255.255.0

vrrp vrid 100 virtual-ip 10.0.10.1

vrrp vrid 100 priority 120

dhcp select global

interface Vlanif110

description ENGINEERS

ip address 10.0.11.2 255.255.255.0

vrrp vrid 110 virtual-ip 10.0.11.1

vrrp vrid 110 priority 120

dhcp select global

interface Vlanif120

ip address 10.0.12.2 255.255.255.0

vrrp vrid 120 virtual-ip 10.0.12.1

interface Vlanif200

description IOT

ip address 10.0.20.2 255.255.255.0

vrrp vrid 200 virtual-ip 10.0.20.1

vrrp vrid 200 priority 120

dhcp select global

interface Vlanif250

description BACKUP

ip address 10.0.21.2 255.255.255.0

vrrp vrid 250 virtual-ip 10.0.21.1

vrrp vrid 250 priority 120

interface MEth0/0/1

interface Eth-Trunk1

description Uplink-to-Core-SW2

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface Eth-Trunk2

description Agg-sw3-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface Eth-Trunk3

description Agg-sw1-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface Eth-Trunk4

traffic-filter inbound acl 3999

interface GigabitEthernet0/0/1

interface GigabitEthernet0/0/2

interface GigabitEthernet0/0/3

interface GigabitEthernet0/0/4

interface GigabitEthernet0/0/5

interface GigabitEthernet0/0/6

interface GigabitEthernet0/0/7

interface GigabitEthernet0/0/8

interface GigabitEthernet0/0/9

interface GigabitEthernet0/0/10

interface GigabitEthernet0/0/11

interface GigabitEthernet0/0/12

interface GigabitEthernet0/0/13

interface GigabitEthernet0/0/14

interface GigabitEthernet0/0/15

eth-trunk 2

interface GigabitEthernet0/0/16

eth-trunk 2

interface GigabitEthernet0/0/17

interface GigabitEthernet0/0/18

interface GigabitEthernet0/0/19

eth-trunk 3

interface GigabitEthernet0/0/20

eth-trunk 3

interface GigabitEthernet0/0/21

interface GigabitEthernet0/0/22

port link-type access

port default vlan 10

interface GigabitEthernet0/0/23

eth-trunk 1

interface GigabitEthernet0/0/24

eth-trunk 1

interface NULL0

ip route-static 0.0.0.0 0.0.0.0 10.0.0.10

ip route-static 10.2.0.0 255.255.0.0 172.16.100.1

ip route-static 172.16.100.0 255.255.255.252 10.0.0.10

stelnet server enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

return

<Core-SW1>

- HQ Core-Sw-2

sysname Core-SW2

FTP server enable

undo info-center enable

vlan batch 10 20 30 40 50 60 70 80 90 100

vlan batch 110 120 200 250

stp mode rstp

stp instance 0 priority 4096

cluster enable

ntdp enable

ndp enable

lldp enable

undo nap slave enable

drop illegal-mac alarm

dhcp enable

diffserv domain default

acl number 3999

rule 5 permit ip source 0.0.0.0 0 destination 255.255.255.255 0

rule 10 permit ip source 10.0.0.0 0.0.0.255 destination 10.0.0.0 0.0.0.255

rule 15 permit ip source 10.0.1.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 20 permit ip source 10.0.2.0 0.0.1.255 destination 10.0.2.0 0.0.1.255

rule 25 permit ip source 10.0.4.0 0.0.0.255 destination 10.0.4.0 0.0.0.255

rule 30 permit ip source 10.0.5.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 35 permit ip source 10.0.6.0 0.0.0.255 destination 10.0.6.0 0.0.0.255

rule 40 permit ip source 10.0.7.0 0.0.0.255 destination 10.0.7.0 0.0.0.255

rule 45 permit ip source 10.0.8.0 0.0.0.255 destination 10.0.8.0 0.0.0.255

rule 50 permit ip source 10.0.9.0 0.0.0.255 destination 10.0.9.0 0.0.0.255

rule 55 permit ip source 10.0.10.0 0.0.0.255 destination 10.0.10.0 0.0.0.255

rule 60 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.11.0 0.0.0.255

rule 65 permit ip source 10.0.12.0 0.0.0.255 destination 10.0.12.0 0.0.0.255

rule 70 permit ip source 10.0.20.0 0.0.0.255 destination 10.0.20.0 0.0.0.255

rule 75 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 80 permit ip source 10.0.0.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 85 permit ip source 10.0.0.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 90 permit ip source 10.0.0.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 95 permit ip source 10.0.2.0 0.0.1.255 destination 10.0.1.0 0.0.0.255

rule 100 permit ip source 10.0.2.0 0.0.1.255 destination 10.0.5.0 0.0.0.255

rule 105 permit ip source 10.0.2.0 0.0.1.255 destination 10.0.21.0 0.0.0.255

rule 110 permit ip source 10.0.4.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 115 permit ip source 10.0.4.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 120 permit ip source 10.0.5.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 125 permit ip source 10.0.5.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 130 permit ip source 10.0.5.0 0.0.0.255 destination 10.0.6.0 0.0.0.255

rule 135 permit ip source 10.0.6.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 140 permit ip source 10.0.6.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 145 permit ip source 10.0.7.0 0.0.0.255 destination 10.0.0.0 0.0.15.255

rule 150 deny ip source 10.0.7.0 0.0.0.255 destination 10.0.10.0 0.0.0.255

rule 155 deny ip source 10.0.7.0 0.0.0.255 destination 10.0.20.0 0.0.0.255

rule 160 deny ip source 10.0.7.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 165 permit ip source 10.0.8.0 0.0.0.255 destination 10.0.12.0 0.0.0.255

rule 170 permit ip source 10.0.9.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 175 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 180 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 185 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 190 permit ip source 10.0.11.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 195 permit ip source 10.0.12.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 200 permit ip source 10.0.20.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 205 permit ip source 10.0.20.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 210 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.2.0 0.0.1.255

rule 215 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 220 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.6.0 0.0.0.255

rule 225 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.9.0 0.0.0.255

rule 230 permit ip source 10.0.21.0 0.0.0.255 destination 10.0.11.0 0.0.0.255

rule 235 deny ip source 10.0.10.0 0.0.0.255 destination 10.0.0.0 0.0.255.255

rule 240 deny ip source 10.0.20.0 0.0.0.255 destination 10.0.0.0 0.0.255.255

rule 245 permit ip source 10.0.1.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 250 permit ip source 10.0.2.0 0.0.1.255 destination 10.2.2.0 0.0.1.255

rule 255 permit ip source 10.0.5.0 0.0.0.255 destination 10.2.6.0 0.0.0.255

rule 260 permit ip source 10.0.11.0 0.0.0.255 destination 10.2.11.0 0.0.0.255

rule 265 permit ip source 10.0.21.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 270 deny ip

drop-profile default

ip pool vlan100-sw2

gateway-list 10.0.10.1

network 10.0.10.0 mask 255.255.255.0

excluded-ip-address 10.0.10.251 10.0.10.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan110-sw2

gateway-list 10.0.11.1

network 10.0.11.0 mask 255.255.255.0

excluded-ip-address 10.0.11.251 10.0.11.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan200-sw2

gateway-list 10.0.20.1

network 10.0.20.0 mask 255.255.255.0

excluded-ip-address 10.0.20.251 10.0.20.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan50-sw2

gateway-list 10.0.5.1

network 10.0.5.0 mask 255.255.255.0

excluded-ip-address 10.0.5.251 10.0.5.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan60-sw2

gateway-list 10.0.6.1

network 10.0.6.0 mask 255.255.255.0

excluded-ip-address 10.0.6.251 10.0.6.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan70-sw2

gateway-list 10.0.7.1

network 10.0.7.0 mask 255.255.255.0

excluded-ip-address 10.0.7.251 10.0.7.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan80-sw2

gateway-list 10.0.8.1

network 10.0.8.0 mask 255.255.255.0

excluded-ip-address 10.0.8.251 10.0.8.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan90-sw2

gateway-list 10.0.9.1

network 10.0.9.0 mask 255.255.255.0

excluded-ip-address 10.0.9.251 10.0.9.254

dns-list 8.8.8.8 1.1.1.1

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user depi password cipher T7O"M%@"'AC=K6%QE'XW;1!!

local-user depi privilege level 10

local-user depi service-type telnet ssh ftp

local-user admin password cipher !<*PMY9[$U)NZPO3JBXBHA!!

local-user admin privilege level 15

local-user admin service-type telnet ssh ftp

local-user viewer password cipher O.]B_,=PL+/J*EP&JX-W=1!!

local-user viewer privilege level 0

local-user viewer service-type telnet ssh ftp

interface Vlanif1

interface Vlanif10

description MGMT

ip address 10.0.0.3 255.255.255.0

vrrp vrid 10 virtual-ip 10.0.0.1

interface Vlanif20

description INFRA

ip address 10.0.1.3 255.255.255.0

vrrp vrid 20 virtual-ip 10.0.1.1

interface Vlanif30

description SERVERS

ip address 10.0.2.3 255.255.254.0

vrrp vrid 30 virtual-ip 10.0.2.1

interface Vlanif40

description DMZ

ip address 10.0.4.3 255.255.255.0

vrrp vrid 40 virtual-ip 10.0.4.1

interface Vlanif50

description CORE_APPS

ip address 10.0.5.3 255.255.255.0

vrrp vrid 50 virtual-ip 10.0.5.1

dhcp select global

interface Vlanif60

description HR

ip address 10.0.6.3 255.255.255.0

vrrp vrid 60 virtual-ip 10.0.6.1

dhcp select global

interface Vlanif70

description IT

ip address 10.0.7.3 255.255.255.0

vrrp vrid 70 virtual-ip 10.0.7.1

dhcp select global

interface Vlanif80

description VOICE

ip address 10.0.8.3 255.255.255.0

vrrp vrid 80 virtual-ip 10.0.8.1

dhcp select global

interface Vlanif90

description CCTV

ip address 10.0.9.3 255.255.255.0

vrrp vrid 90 virtual-ip 10.0.9.1

dhcp select global

interface Vlanif100

description GUEST

ip address 10.0.10.3 255.255.255.0

vrrp vrid 100 virtual-ip 10.0.10.1

dhcp select global

interface Vlanif110

description ENGINEERS

ip address 10.0.11.3 255.255.255.0

vrrp vrid 110 virtual-ip 10.0.11.1

dhcp select global

interface Vlanif120

ip address 10.0.12.3 255.255.255.0

vrrp vrid 120 virtual-ip 10.0.12.1

interface Vlanif200

description IOT

ip address 10.0.20.3 255.255.255.0

vrrp vrid 200 virtual-ip 10.0.20.1

dhcp select global

interface Vlanif250

ip address 10.0.21.3 255.255.255.0

vrrp vrid 250 virtual-ip 10.0.21.1

interface MEth0/0/1

interface Eth-Trunk1

description Uplink-to-Core-SW1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface Eth-Trunk2

description Agg-sw3-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface Eth-Trunk3

description Agg-sw1-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface GigabitEthernet0/0/1

interface GigabitEthernet0/0/2

interface GigabitEthernet0/0/3

interface GigabitEthernet0/0/4

interface GigabitEthernet0/0/5

interface GigabitEthernet0/0/6

interface GigabitEthernet0/0/7

interface GigabitEthernet0/0/8

interface GigabitEthernet0/0/9

interface GigabitEthernet0/0/10

interface GigabitEthernet0/0/11

interface GigabitEthernet0/0/12

interface GigabitEthernet0/0/13

interface GigabitEthernet0/0/14

interface GigabitEthernet0/0/15

eth-trunk 3

interface GigabitEthernet0/0/16

eth-trunk 3

interface GigabitEthernet0/0/17

interface GigabitEthernet0/0/18

interface GigabitEthernet0/0/19

eth-trunk 2

interface GigabitEthernet0/0/20

eth-trunk 2

interface GigabitEthernet0/0/21

interface GigabitEthernet0/0/22

port link-type access

port default vlan 10

interface GigabitEthernet0/0/23

eth-trunk 1

interface GigabitEthernet0/0/24

eth-trunk 1

interface NULL0

ip route-static 0.0.0.0 0.0.0.0 10.0.0.10

ip route-static 10.2.0.0 255.255.0.0 10.0.0.10

ip route-static 10.2.0.0 255.255.0.0 172.16.100.1

ip route-static 172.16.100.0 255.255.255.252 10.0.0.10

stelnet server enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

return

- HQ Agg-sw-1

sysname Agg-sw1

undo info-center enable

vlan batch 10 20 30 40 50 60 70 80 90 100

vlan batch 110 120 200 250

stp mode rstp

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

dhcp enable

dhcp snooping enable

diffserv domain default

drop-profile default

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password simple admin

local-user admin service-type http

interface Vlanif1

interface MEth0/0/1

interface Eth-Trunk1

description Core-sw-1-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

stp edged-port disable

dhcp snooping trusted

interface Eth-Trunk2

description Core-sw-2-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

stp edged-port disable

dhcp snooping trusted

interface GigabitEthernet0/0/1

port link-type access

port default vlan 70

interface GigabitEthernet0/0/2

port link-type access

port default vlan 60

interface GigabitEthernet0/0/3

port link-type access

port default vlan 200

interface GigabitEthernet0/0/4

interface GigabitEthernet0/0/5

interface GigabitEthernet0/0/6

interface GigabitEthernet0/0/7

interface GigabitEthernet0/0/8

interface GigabitEthernet0/0/9

interface GigabitEthernet0/0/10

interface GigabitEthernet0/0/11

interface GigabitEthernet0/0/12

interface GigabitEthernet0/0/13

interface GigabitEthernet0/0/14

interface GigabitEthernet0/0/15

interface GigabitEthernet0/0/16

interface GigabitEthernet0/0/17

interface GigabitEthernet0/0/18

interface GigabitEthernet0/0/19

interface GigabitEthernet0/0/20

interface GigabitEthernet0/0/21

eth-trunk 2

interface GigabitEthernet0/0/22

eth-trunk 2

interface GigabitEthernet0/0/23

eth-trunk 1

interface GigabitEthernet0/0/24

eth-trunk 1

interface NULL0

user-interface con 0

user-interface vty 0 4

return

- HQ-Agg-sw-2

sysname Agg-SW2

undo info-center enable

vlan batch 10 20 30 40 50 60 70 80 90 100

vlan batch 110 120 200 250

stp mode rstp

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

dhcp enable

dhcp snooping enable

diffserv domain default

drop-profile default

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password simple admin

local-user admin service-type http

interface Vlanif1

interface MEth0/0/1

interface Eth-Trunk1

description Core-sw-1-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

stp edged-port disable

dhcp snooping trusted

interface Eth-Trunk2

description Core-sw-2-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

stp edged-port disable

dhcp snooping trusted

interface GigabitEthernet0/0/1

port link-type access

port default vlan 70

interface GigabitEthernet0/0/2

port link-type access

port default vlan 110

interface GigabitEthernet0/0/3

port link-type access

port default vlan 60

interface GigabitEthernet0/0/4

interface GigabitEthernet0/0/5

interface GigabitEthernet0/0/6

interface GigabitEthernet0/0/7

interface GigabitEthernet0/0/8

interface GigabitEthernet0/0/9

interface GigabitEthernet0/0/10

interface GigabitEthernet0/0/11

interface GigabitEthernet0/0/12

interface GigabitEthernet0/0/13

interface GigabitEthernet0/0/14

interface GigabitEthernet0/0/15

interface GigabitEthernet0/0/16

interface GigabitEthernet0/0/17

interface GigabitEthernet0/0/18

interface GigabitEthernet0/0/19

interface GigabitEthernet0/0/20

interface GigabitEthernet0/0/21

eth-trunk 2

interface GigabitEthernet0/0/22

eth-trunk 2

interface GigabitEthernet0/0/23

eth-trunk 1

interface GigabitEthernet0/0/24

eth-trunk 1

interface NULL0

user-interface con 0

user-interface vty 0 4

return

- HQ-Agg-sw-3

sysname Agg-SW3

undo info-center enable

vlan batch 10 20 30 40 50 60 70 80 90 100

vlan batch 110 120 200 250

stp mode rstp

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

dhcp enable

dhcp snooping enable

diffserv domain default

drop-profile default

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password simple admin

local-user admin service-type http

interface Vlanif1

interface MEth0/0/1

interface Eth-Trunk1

description Core-sw1-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

stp edged-port disable

dhcp snooping trusted

interface Eth-Trunk2

description Core-sw2-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

dhcp snooping trusted

interface Eth-Trunk3

description AP-UPLINK

port link-type trunk

port trunk allow-pass vlan 100 120

interface GigabitEthernet0/0/1

description File-Storage-UPLINK

port link-type access

port default vlan 20

interface GigabitEthernet0/0/2

description NVR-UPLINK

port link-type access

port default vlan 90

interface GigabitEthernet0/0/3

description VOIP-UPLINK

port link-type access

port default vlan 80

interface GigabitEthernet0/0/4

port link-type access

port default vlan 70

interface GigabitEthernet0/0/5

interface GigabitEthernet0/0/6

interface GigabitEthernet0/0/7

interface GigabitEthernet0/0/8

interface GigabitEthernet0/0/9

interface GigabitEthernet0/0/10

interface GigabitEthernet0/0/11

interface GigabitEthernet0/0/12

interface GigabitEthernet0/0/13

interface GigabitEthernet0/0/14

interface GigabitEthernet0/0/15

interface GigabitEthernet0/0/16

interface GigabitEthernet0/0/17

interface GigabitEthernet0/0/18

description DHCP-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

interface GigabitEthernet0/0/19

eth-trunk 3

interface GigabitEthernet0/0/20

eth-trunk 3

interface GigabitEthernet0/0/21

eth-trunk 2

interface GigabitEthernet0/0/22

eth-trunk 2

interface GigabitEthernet0/0/23

eth-trunk 1

interface GigabitEthernet0/0/24

eth-trunk 1

interface NULL0

user-interface con 0

user-interface vty 0 4

return

- HQ-AC-1

sysname AC-HQ

undo http secure-server enable

set memory-usage threshold 0

ssl renegotiation-rate 1

vlan batch 100 120

authentication-profile name default_authen_profile

authentication-profile name dot1x_authen_profile

authentication-profile name mac_authen_profile

authentication-profile name portal_authen_profile

authentication-profile name macportal_authen_profile

dhcp enable

dhcp snooping enable

diffserv domain default

radius-server template default

pki realm default

rsa local-key-pair default

enrollment self-signed

ike proposal default

encryption-algorithm aes-256

dh group14

authentication-algorithm sha2-256

authentication-method pre-share

integrity-algorithm hmac-sha2-256

prf hmac-sha2-256

free-rule-template name default_free_rule

portal-access-profile name portal_access_profile

ip pool vlan120

gateway-list 10.0.12.1

network 10.0.12.0 mask 255.255.255.0

dns-list 8.8.8.8 8.8.4.4

aaa

authentication-scheme default

authentication-scheme radius

authentication-mode radius

authorization-scheme default

accounting-scheme default

domain default

authentication-scheme radius

radius-server default

domain default_admin

authentication-scheme default

local-user admin password irreversible-cipher $1a$7(vSY/0I*,$zcpkA=T\W6Vtum2'.y}O8bDuT].C&$*;,d8@I*5S$

local-user admin service-type http

interface Vlanif120

ip address 10.0.12.252 255.255.255.0

dhcp select global

interface Eth-Trunk1

description AGG-SW3-UPLINK

port link-type trunk

port trunk allow-pass vlan 100 120

dhcp snooping trusted

interface Eth-Trunk2

description AP-1-UPLINK

port link-type trunk

port trunk pvid vlan 120

port trunk allow-pass vlan 100 120

interface Eth-Trunk3

description AP-2-UPLINK

port link-type trunk

port trunk pvid vlan 120

port trunk allow-pass vlan 100 120

interface Eth-Trunk4

description AP-3-UPLINK

port link-type trunk

port trunk pvid vlan 120

port trunk allow-pass vlan 100 120

interface GigabitEthernet0/0/1

eth-trunk 4

interface GigabitEthernet0/0/2

eth-trunk 4

interface GigabitEthernet0/0/3

eth-trunk 3

interface GigabitEthernet0/0/4

eth-trunk 3

interface GigabitEthernet0/0/5

eth-trunk 2

interface GigabitEthernet0/0/6

eth-trunk 2

interface GigabitEthernet0/0/7

undo negotiation auto

duplex half

eth-trunk 1

interface GigabitEthernet0/0/8

undo negotiation auto

duplex half

eth-trunk 1

interface NULL0

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

ssh server secure-algorithms cipher aes256_ctr aes128_ctr

ssh server key-exchange dh_group14_sha1

ssh client secure-algorithms cipher aes256_ctr aes128_ctr

ssh client secure-algorithms hmac sha2_256

ssh client key-exchange dh_group14_sha1

ip route-static 0.0.0.0 0.0.0.0 10.0.12.1

capwap source ip-address 10.0.12.252

user-interface con 0

user-interface vty 0 4

protocol inbound all

user-interface vty 16 20

protocol inbound all

wlan

traffic-profile name default

security-profile name default

security-profile name default-wds

security-profile name Corelink-Sec

security wpa-wpa2 psk pass-phrase %^%#;ov#(gw+_,h!FeM#1SSI(#DWGrV(o1Uhxm&K!yoT%^%# aes

security-profile name default-mesh

ssid-profile name default

ssid-profile name Corelink-SSID

ssid "Corelink Technologies"

vap-profile name default

vap-profile name Corelink-VAP

service-vlan vlan-id 100

ssid-profile Corelink-SSID

security-profile Corelink-Sec

wds-profile name default

mesh-handover-profile name default

mesh-profile name default

regulatory-domain-profile name default

air-scan-profile name default

rrm-profile name default

radio-2g-profile name default

radio-5g-profile name default

wids-spoof-profile name default

wids-profile name default

wireless-access-specification

ap-system-profile name default

port-link-profile name default

wired-port-profile name default

serial-profile name preset-enjoyor-toeap

ap-group name default

ap-group name CORELINK-APs

radio 0

vap-profile Corelink-VAP wlan 1

radio 1

vap-profile Corelink-VAP wlan 1

ap-id 1 type-id 69 ap-mac 00e0-fc69-4940 ap-sn 2102354483102A1F6A74

ap-name Corelink-AP1

ap-group CORELINK-APs

ap-id 2 type-id 69 ap-mac 00e0-fcd4-63e0 ap-sn 2102354483105A412F21

ap-name Corelink-AP2

ap-group CORELINK-APs

ap-id 3 type-id 69 ap-mac 00e0-fcba-76d0 ap-sn 2102354483108C3A7737

ap-name Corelink-AP3

ap-group CORELINK-APs

provision-ap

dot1x-access-profile name dot1x_access_profile

mac-access-profile name mac_access_profile

return

-------------------------------------------------------------------------------------------
-------------------------------------------------------------------------------------------

- Branch main router

sysname AC-HQ

undo http secure-server enable

set memory-usage threshold 0

ssl renegotiation-rate 1

vlan batch 100 120

authentication-profile name default_authen_profile

authentication-profile name dot1x_authen_profile

authentication-profile name mac_authen_profile

authentication-profile name portal_authen_profile

authentication-profile name macportal_authen_profile

dhcp enable

dhcp snooping enable

diffserv domain default

radius-server template default

pki realm default

rsa local-key-pair default

enrollment self-signed

ike proposal default

encryption-algorithm aes-256

dh group14

authentication-algorithm sha2-256

authentication-method pre-share

integrity-algorithm hmac-sha2-256

prf hmac-sha2-256

free-rule-template name default_free_rule

portal-access-profile name portal_access_profile

ip pool vlan120

gateway-list 10.0.12.1

network 10.0.12.0 mask 255.255.255.0

dns-list 8.8.8.8 8.8.4.4

aaa

authentication-scheme default

authentication-scheme radius

authentication-mode radius

authorization-scheme default

accounting-scheme default

domain default

authentication-scheme radius

radius-server default

domain default_admin

authentication-scheme default

local-user admin password irreversible-cipher $1a$7(vSY/0I*,$zcpkA=T\W6Vtum2'.y}O8bDuT].C&$*;,d8@I*5S$

local-user admin service-type http

interface Vlanif120

ip address 10.0.12.252 255.255.255.0

dhcp select global

interface Eth-Trunk1

description AGG-SW3-UPLINK

port link-type trunk

port trunk allow-pass vlan 100 120

dhcp snooping trusted

interface Eth-Trunk2

description AP-1-UPLINK

port link-type trunk

port trunk pvid vlan 120

port trunk allow-pass vlan 100 120

interface Eth-Trunk3

description AP-2-UPLINK

port link-type trunk

port trunk pvid vlan 120

port trunk allow-pass vlan 100 120

interface Eth-Trunk4

description AP-3-UPLINK

port link-type trunk

port trunk pvid vlan 120

port trunk allow-pass vlan 100 120

interface GigabitEthernet0/0/1

eth-trunk 4

interface GigabitEthernet0/0/2

eth-trunk 4

interface GigabitEthernet0/0/3

eth-trunk 3

interface GigabitEthernet0/0/4

eth-trunk 3

interface GigabitEthernet0/0/5

eth-trunk 2

interface GigabitEthernet0/0/6

eth-trunk 2

interface GigabitEthernet0/0/7

undo negotiation auto

duplex half

eth-trunk 1

interface GigabitEthernet0/0/8

undo negotiation auto

duplex half

eth-trunk 1

interface NULL0

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

ssh server secure-algorithms cipher aes256_ctr aes128_ctr

ssh server key-exchange dh_group14_sha1

ssh client secure-algorithms cipher aes256_ctr aes128_ctr

ssh client secure-algorithms hmac sha2_256

ssh client key-exchange dh_group14_sha1

ip route-static 0.0.0.0 0.0.0.0 10.0.12.1

capwap source ip-address 10.0.12.252

user-interface con 0

user-interface vty 0 4

protocol inbound all

user-interface vty 16 20

protocol inbound all

wlan

traffic-profile name default

security-profile name default

security-profile name default-wds

security-profile name Corelink-Sec

security wpa-wpa2 psk pass-phrase %^%#;ov#(gw+_,h!FeM#1SSI(#DWGrV(o1Uhxm&K!yoT%^%# aes

security-profile name default-mesh

ssid-profile name default

ssid-profile name Corelink-SSID

ssid "Corelink Technologies"

vap-profile name default

vap-profile name Corelink-VAP

service-vlan vlan-id 100

ssid-profile Corelink-SSID

security-profile Corelink-Sec

wds-profile name default

mesh-handover-profile name default

mesh-profile name default

regulatory-domain-profile name default

air-scan-profile name default

rrm-profile name default

radio-2g-profile name default

radio-5g-profile name default

wids-spoof-profile name default

wids-profile name default

wireless-access-specification

ap-system-profile name default

port-link-profile name default

wired-port-profile name default

serial-profile name preset-enjoyor-toeap

ap-group name default

ap-group name CORELINK-APs

radio 0

vap-profile Corelink-VAP wlan 1

radio 1

vap-profile Corelink-VAP wlan 1

ap-id 1 type-id 69 ap-mac 00e0-fc69-4940 ap-sn 2102354483102A1F6A74

ap-name Corelink-AP1

ap-group CORELINK-APs

ap-id 2 type-id 69 ap-mac 00e0-fcd4-63e0 ap-sn 2102354483105A412F21

ap-name Corelink-AP2

ap-group CORELINK-APs

ap-id 3 type-id 69 ap-mac 00e0-fcba-76d0 ap-sn 2102354483108C3A7737

ap-name Corelink-AP3

ap-group CORELINK-APs

provision-ap

dot1x-access-profile name dot1x_access_profile

mac-access-profile name mac_access_profile

return

- BR1-core-sw-1

sysname BR1-Coreswitch-2

FTP server enable

undo info-center enable

vlan batch 10 20 30 40 50 60 70 80 90 100

vlan batch 110 120 200 250

stp mode rstp

stp instance 0 priority 0

cluster enable

ntdp enable

ndp enable

undo nap slave enable

drop illegal-mac alarm

dhcp enable

diffserv domain default

acl number 3999

rule 5 permit ip source 0.0.0.0 0 destination 255.255.255.255 0

rule 10 permit ip source 10.2.0.0 0.0.0.255 destination 10.2.0.0 0.0.0.255

rule 15 permit ip source 10.2.1.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 20 permit ip source 10.2.2.0 0.0.1.255 destination 10.2.2.0 0.0.1.255

rule 25 permit ip source 10.2.4.0 0.0.0.255 destination 10.2.4.0 0.0.0.255

rule 30 permit ip source 10.2.5.0 0.0.0.255 destination 10.2.5.0 0.0.0.255

rule 35 permit ip source 10.2.6.0 0.0.0.255 destination 10.2.6.0 0.0.0.255

rule 40 permit ip source 10.2.7.0 0.0.0.255 destination 10.2.7.0 0.0.0.255

rule 45 permit ip source 10.2.8.0 0.0.0.255 destination 10.2.8.0 0.0.0.255

rule 50 permit ip source 10.2.9.0 0.0.0.255 destination 10.2.9.0 0.0.0.255

rule 55 permit ip source 10.2.10.0 0.0.0.255 destination 10.2.10.0 0.0.0.255

rule 60 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.11.0 0.0.0.255

rule 65 permit ip source 10.2.12.0 0.0.0.255 destination 10.2.12.0 0.0.0.255

rule 70 permit ip source 10.2.20.0 0.0.0.255 destination 10.2.20.0 0.0.0.255

rule 75 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 80 permit ip source 10.2.0.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 85 permit ip source 10.2.0.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 90 permit ip source 10.2.0.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 95 permit ip source 10.2.2.0 0.0.1.255 destination 10.2.1.0 0.0.0.255

rule 100 permit ip source 10.2.2.0 0.0.1.255 destination 10.2.5.0 0.0.0.255

rule 105 permit ip source 10.2.2.0 0.0.1.255 destination 10.2.21.0 0.0.0.255

rule 110 permit ip source 10.2.4.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 115 permit ip source 10.2.4.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 120 permit ip source 10.2.5.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 125 permit ip source 10.2.5.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 130 permit ip source 10.2.5.0 0.0.0.255 destination 10.2.6.0 0.0.0.255

rule 135 permit ip source 10.2.6.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 140 permit ip source 10.2.6.0 0.0.0.255 destination 10.2.5.0 0.0.0.255

rule 145 permit ip source 10.2.7.0 0.0.0.255 destination 10.2.0.0 0.0.31.255

rule 150 deny ip source 10.2.7.0 0.0.0.255 destination 10.2.10.0 0.0.0.255

rule 155 deny ip source 10.2.7.0 0.0.0.255 destination 10.2.20.0 0.0.0.255

rule 160 deny ip source 10.2.7.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 165 permit ip source 10.2.8.0 0.0.0.255 destination 10.2.12.0 0.0.0.255

rule 170 permit ip source 10.2.9.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 175 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 180 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 185 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.5.0 0.0.0.255

rule 190 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 195 permit ip source 10.2.12.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 200 permit ip source 10.2.20.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 205 permit ip source 10.2.20.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 210 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 215 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.5.0 0.0.0.255

rule 220 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.6.0 0.0.0.255

rule 225 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.9.0 0.0.0.255

rule 230 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.11.0 0.0.0.255

rule 235 deny ip source 10.2.10.0 0.0.0.255 destination 10.2.0.0 0.0.255.255

rule 240 deny ip source 10.2.20.0 0.0.0.255 destination 10.2.0.0 0.0.255.255

rule 245 deny ip source 10.2.11.0 0.0.0.255 destination 10.2.10.0 0.0.0.255

rule 250 permit ip source 10.2.1.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 255 permit ip source 10.2.2.0 0.0.1.255 destination 10.0.2.0 0.0.1.255

rule 260 permit ip source 10.2.6.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 265 permit ip source 10.2.11.0 0.0.0.255 destination 10.0.11.0 0.0.0.255

rule 270 permit ip source 10.2.21.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 275 deny ip

drop-profile default

ip pool vlan100-sw2

gateway-list 10.2.10.1

network 10.2.10.0 mask 255.255.255.0

excluded-ip-address 10.2.10.251 10.2.10.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan110-sw2

gateway-list 10.2.11.1

network 10.2.11.0 mask 255.255.255.0

excluded-ip-address 10.2.11.251 10.2.11.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan200-sw2

gateway-list 10.2.20.1

network 10.2.20.0 mask 255.255.255.0

excluded-ip-address 10.2.20.251 10.2.20.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan50-sw2

gateway-list 10.2.5.1

network 10.2.5.0 mask 255.255.255.0

excluded-ip-address 10.2.5.251 10.2.5.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan60-sw2

gateway-list 10.2.6.1

network 10.2.6.0 mask 255.255.255.0

excluded-ip-address 10.2.6.251 10.2.6.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan70-sw2

gateway-list 10.2.7.1

network 10.2.7.0 mask 255.255.255.0

excluded-ip-address 10.2.7.251 10.2.7.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan80-sw2

gateway-list 10.2.8.1

network 10.2.8.0 mask 255.255.255.0

excluded-ip-address 10.2.8.251 10.2.8.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan90-sw2

gateway-list 10.2.9.1

network 10.2.9.0 mask 255.255.255.0

excluded-ip-address 10.2.9.251 10.2.9.254

dns-list 8.8.8.8 1.1.1.1

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user depi password cipher T7O"M%@"'AC=K6%QE'XW;1!!

local-user depi privilege level 10

local-user depi service-type telnet ssh ftp

local-user admin password cipher !<*PMY9[$U)NZPO3JBXBHA!!

local-user admin privilege level 15

local-user admin service-type telnet ssh ftp

local-user viewer password cipher O.]B_,=PL+/J*EP&JX-W=1!!

local-user viewer privilege level 0

local-user viewer service-type telnet ssh ftp

interface Vlanif1

interface Vlanif10

description MGMT

ip address 10.2.0.3 255.255.255.0

vrrp vrid 10 virtual-ip 10.2.0.1

dhcp select global

interface Vlanif20

description INFRA

ip address 10.2.1.3 255.255.255.0

vrrp vrid 20 virtual-ip 10.2.1.1

dhcp select global

interface Vlanif30

description SERVERS

ip address 10.2.2.3 255.255.254.0

vrrp vrid 30 virtual-ip 10.2.2.1

dhcp select global

interface Vlanif40

description DMZ

ip address 10.2.4.3 255.255.255.0

vrrp vrid 40 virtual-ip 10.2.4.1

dhcp select global

interface Vlanif50

description CORE_APPS

ip address 10.2.5.3 255.255.255.0

vrrp vrid 50 virtual-ip 10.2.5.1

dhcp select global

interface Vlanif60

description HR

ip address 10.2.6.3 255.255.255.0

vrrp vrid 60 virtual-ip 10.2.6.1

dhcp select global

interface Vlanif70

description IT

ip address 10.2.7.3 255.255.255.0

vrrp vrid 70 virtual-ip 10.2.7.1

dhcp select global

dhcp select relay

interface Vlanif80

description VOICE

ip address 10.2.8.3 255.255.255.0

vrrp vrid 80 virtual-ip 10.2.8.1

dhcp select global

interface Vlanif90

description CCTV

ip address 10.2.9.3 255.255.255.0

vrrp vrid 90 virtual-ip 10.2.9.1

dhcp select global

interface Vlanif100

description GUEST

ip address 10.2.10.3 255.255.255.0

vrrp vrid 100 virtual-ip 10.2.10.1

dhcp select global

interface Vlanif110

description ENGINEERS

ip address 10.2.11.3 255.255.255.0

vrrp vrid 110 virtual-ip 10.2.11.1

dhcp select global

interface Vlanif120

ip address 10.2.12.3 255.255.255.0

vrrp vrid 120 virtual-ip 10.2.12.1

interface Vlanif200

description IOT

ip address 10.2.20.3 255.255.255.0

vrrp vrid 200 virtual-ip 10.2.20.1

dhcp select global

interface Vlanif250

description BACKUP

ip address 10.2.21.3 255.255.255.0

vrrp vrid 250 virtual-ip 10.2.21.1

interface MEth0/0/1

interface Eth-Trunk1

description Uplink-to-Core-SW1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface Eth-Trunk2

description Agg-sw2-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface Eth-Trunk3

description Agg-sw1-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface GigabitEthernet0/0/1

interface GigabitEthernet0/0/2

interface GigabitEthernet0/0/3

interface GigabitEthernet0/0/4

interface GigabitEthernet0/0/5

interface GigabitEthernet0/0/6

interface GigabitEthernet0/0/7

interface GigabitEthernet0/0/8

interface GigabitEthernet0/0/9

interface GigabitEthernet0/0/10

interface GigabitEthernet0/0/11

interface GigabitEthernet0/0/12

interface GigabitEthernet0/0/13

interface GigabitEthernet0/0/14

interface GigabitEthernet0/0/15

interface GigabitEthernet0/0/16

port link-type access

port default vlan 10

interface GigabitEthernet0/0/17

eth-trunk 3

interface GigabitEthernet0/0/18

eth-trunk 3

interface GigabitEthernet0/0/19

eth-trunk 2

interface GigabitEthernet0/0/20

eth-trunk 2

interface GigabitEthernet0/0/21

eth-trunk 1

interface GigabitEthernet0/0/22

eth-trunk 1

interface GigabitEthernet0/0/23

interface GigabitEthernet0/0/24

interface NULL0

ip route-static 0.0.0.0 0.0.0.0 10.2.0.10

ip route-static 10.0.0.0 255.255.0.0 10.2.0.10

ip route-static 10.0.0.0 255.255.0.0 172.16.100.2

ip route-static 172.16.100.0 255.255.255.252 10.2.0.10

stelnet server enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

return

- BR1-core-sw-2

sysname BR1-Coreswitch-2

FTP server enable

undo info-center enable

vlan batch 10 20 30 40 50 60 70 80 90 100

vlan batch 110 120 200 250

stp mode rstp

stp instance 0 priority 0

cluster enable

ntdp enable

ndp enable

undo nap slave enable

drop illegal-mac alarm

dhcp enable

diffserv domain default

acl number 3999

rule 5 permit ip source 0.0.0.0 0 destination 255.255.255.255 0

rule 10 permit ip source 10.2.0.0 0.0.0.255 destination 10.2.0.0 0.0.0.255

rule 15 permit ip source 10.2.1.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 20 permit ip source 10.2.2.0 0.0.1.255 destination 10.2.2.0 0.0.1.255

rule 25 permit ip source 10.2.4.0 0.0.0.255 destination 10.2.4.0 0.0.0.255

rule 30 permit ip source 10.2.5.0 0.0.0.255 destination 10.2.5.0 0.0.0.255

rule 35 permit ip source 10.2.6.0 0.0.0.255 destination 10.2.6.0 0.0.0.255

rule 40 permit ip source 10.2.7.0 0.0.0.255 destination 10.2.7.0 0.0.0.255

rule 45 permit ip source 10.2.8.0 0.0.0.255 destination 10.2.8.0 0.0.0.255

rule 50 permit ip source 10.2.9.0 0.0.0.255 destination 10.2.9.0 0.0.0.255

rule 55 permit ip source 10.2.10.0 0.0.0.255 destination 10.2.10.0 0.0.0.255

rule 60 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.11.0 0.0.0.255

rule 65 permit ip source 10.2.12.0 0.0.0.255 destination 10.2.12.0 0.0.0.255

rule 70 permit ip source 10.2.20.0 0.0.0.255 destination 10.2.20.0 0.0.0.255

rule 75 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 80 permit ip source 10.2.0.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 85 permit ip source 10.2.0.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 90 permit ip source 10.2.0.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 95 permit ip source 10.2.2.0 0.0.1.255 destination 10.2.1.0 0.0.0.255

rule 100 permit ip source 10.2.2.0 0.0.1.255 destination 10.2.5.0 0.0.0.255

rule 105 permit ip source 10.2.2.0 0.0.1.255 destination 10.2.21.0 0.0.0.255

rule 110 permit ip source 10.2.4.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 115 permit ip source 10.2.4.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 120 permit ip source 10.2.5.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 125 permit ip source 10.2.5.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 130 permit ip source 10.2.5.0 0.0.0.255 destination 10.2.6.0 0.0.0.255

rule 135 permit ip source 10.2.6.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 140 permit ip source 10.2.6.0 0.0.0.255 destination 10.2.5.0 0.0.0.255

rule 145 permit ip source 10.2.7.0 0.0.0.255 destination 10.2.0.0 0.0.31.255

rule 150 deny ip source 10.2.7.0 0.0.0.255 destination 10.2.10.0 0.0.0.255

rule 155 deny ip source 10.2.7.0 0.0.0.255 destination 10.2.20.0 0.0.0.255

rule 160 deny ip source 10.2.7.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 165 permit ip source 10.2.8.0 0.0.0.255 destination 10.2.12.0 0.0.0.255

rule 170 permit ip source 10.2.9.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 175 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 180 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 185 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.5.0 0.0.0.255

rule 190 permit ip source 10.2.11.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 195 permit ip source 10.2.12.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 200 permit ip source 10.2.20.0 0.0.0.255 destination 10.2.1.0 0.0.0.255

rule 205 permit ip source 10.2.20.0 0.0.0.255 destination 10.2.21.0 0.0.0.255

rule 210 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.2.0 0.0.1.255

rule 215 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.5.0 0.0.0.255

rule 220 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.6.0 0.0.0.255

rule 225 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.9.0 0.0.0.255

rule 230 permit ip source 10.2.21.0 0.0.0.255 destination 10.2.11.0 0.0.0.255

rule 235 deny ip source 10.2.10.0 0.0.0.255 destination 10.2.0.0 0.0.255.255

rule 240 deny ip source 10.2.20.0 0.0.0.255 destination 10.2.0.0 0.0.255.255

rule 245 deny ip source 10.2.11.0 0.0.0.255 destination 10.2.10.0 0.0.0.255

rule 250 permit ip source 10.2.1.0 0.0.0.255 destination 10.0.1.0 0.0.0.255

rule 255 permit ip source 10.2.2.0 0.0.1.255 destination 10.0.2.0 0.0.1.255

rule 260 permit ip source 10.2.6.0 0.0.0.255 destination 10.0.5.0 0.0.0.255

rule 265 permit ip source 10.2.11.0 0.0.0.255 destination 10.0.11.0 0.0.0.255

rule 270 permit ip source 10.2.21.0 0.0.0.255 destination 10.0.21.0 0.0.0.255

rule 275 deny ip

drop-profile default

ip pool vlan100-sw2

gateway-list 10.2.10.1

network 10.2.10.0 mask 255.255.255.0

excluded-ip-address 10.2.10.251 10.2.10.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan110-sw2

gateway-list 10.2.11.1

network 10.2.11.0 mask 255.255.255.0

excluded-ip-address 10.2.11.251 10.2.11.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan200-sw2

gateway-list 10.2.20.1

network 10.2.20.0 mask 255.255.255.0

excluded-ip-address 10.2.20.251 10.2.20.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan50-sw2

gateway-list 10.2.5.1

network 10.2.5.0 mask 255.255.255.0

excluded-ip-address 10.2.5.251 10.2.5.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan60-sw2

gateway-list 10.2.6.1

network 10.2.6.0 mask 255.255.255.0

excluded-ip-address 10.2.6.251 10.2.6.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan70-sw2

gateway-list 10.2.7.1

network 10.2.7.0 mask 255.255.255.0

excluded-ip-address 10.2.7.251 10.2.7.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan80-sw2

gateway-list 10.2.8.1

network 10.2.8.0 mask 255.255.255.0

excluded-ip-address 10.2.8.251 10.2.8.254

dns-list 8.8.8.8 1.1.1.1

ip pool vlan90-sw2

gateway-list 10.2.9.1

network 10.2.9.0 mask 255.255.255.0

excluded-ip-address 10.2.9.251 10.2.9.254

dns-list 8.8.8.8 1.1.1.1

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user depi password cipher T7O"M%@"'AC=K6%QE'XW;1!!

local-user depi privilege level 10

local-user depi service-type telnet ssh ftp

local-user admin password cipher !<*PMY9[$U)NZPO3JBXBHA!!

local-user admin privilege level 15

local-user admin service-type telnet ssh ftp

local-user viewer password cipher O.]B_,=PL+/J*EP&JX-W=1!!

local-user viewer privilege level 0

local-user viewer service-type telnet ssh ftp

interface Vlanif1

interface Vlanif10

description MGMT

ip address 10.2.0.3 255.255.255.0

vrrp vrid 10 virtual-ip 10.2.0.1

dhcp select global

interface Vlanif20

description INFRA

ip address 10.2.1.3 255.255.255.0

vrrp vrid 20 virtual-ip 10.2.1.1

dhcp select global

interface Vlanif30

description SERVERS

ip address 10.2.2.3 255.255.254.0

vrrp vrid 30 virtual-ip 10.2.2.1

dhcp select global

interface Vlanif40

description DMZ

ip address 10.2.4.3 255.255.255.0

vrrp vrid 40 virtual-ip 10.2.4.1

dhcp select global

interface Vlanif50

description CORE_APPS

ip address 10.2.5.3 255.255.255.0

vrrp vrid 50 virtual-ip 10.2.5.1

dhcp select global

interface Vlanif60

description HR

ip address 10.2.6.3 255.255.255.0

vrrp vrid 60 virtual-ip 10.2.6.1

dhcp select global

interface Vlanif70

description IT

ip address 10.2.7.3 255.255.255.0

vrrp vrid 70 virtual-ip 10.2.7.1

dhcp select global

dhcp select relay

interface Vlanif80

description VOICE

ip address 10.2.8.3 255.255.255.0

vrrp vrid 80 virtual-ip 10.2.8.1

dhcp select global

interface Vlanif90

description CCTV

ip address 10.2.9.3 255.255.255.0

vrrp vrid 90 virtual-ip 10.2.9.1

dhcp select global

interface Vlanif100

description GUEST

ip address 10.2.10.3 255.255.255.0

vrrp vrid 100 virtual-ip 10.2.10.1

dhcp select global

interface Vlanif110

description ENGINEERS

ip address 10.2.11.3 255.255.255.0

vrrp vrid 110 virtual-ip 10.2.11.1

dhcp select global

interface Vlanif120

ip address 10.2.12.3 255.255.255.0

vrrp vrid 120 virtual-ip 10.2.12.1

interface Vlanif200

description IOT

ip address 10.2.20.3 255.255.255.0

vrrp vrid 200 virtual-ip 10.2.20.1

dhcp select global

interface Vlanif250

description BACKUP

ip address 10.2.21.3 255.255.255.0

vrrp vrid 250 virtual-ip 10.2.21.1

interface MEth0/0/1

interface Eth-Trunk1

description Uplink-to-Core-SW1

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface Eth-Trunk2

description Agg-sw2-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface Eth-Trunk3

description Agg-sw1-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

traffic-filter inbound acl 3999

interface GigabitEthernet0/0/1

interface GigabitEthernet0/0/2

interface GigabitEthernet0/0/3

interface GigabitEthernet0/0/4

interface GigabitEthernet0/0/5

interface GigabitEthernet0/0/6

interface GigabitEthernet0/0/7

interface GigabitEthernet0/0/8

interface GigabitEthernet0/0/9

interface GigabitEthernet0/0/10

interface GigabitEthernet0/0/11

interface GigabitEthernet0/0/12

interface GigabitEthernet0/0/13

interface GigabitEthernet0/0/14

interface GigabitEthernet0/0/15

interface GigabitEthernet0/0/16

port link-type access

port default vlan 10

interface GigabitEthernet0/0/17

eth-trunk 3

interface GigabitEthernet0/0/18

eth-trunk 3

interface GigabitEthernet0/0/19

eth-trunk 2

interface GigabitEthernet0/0/20

eth-trunk 2

interface GigabitEthernet0/0/21

eth-trunk 1

interface GigabitEthernet0/0/22

eth-trunk 1

interface GigabitEthernet0/0/23

interface GigabitEthernet0/0/24

interface NULL0

ip route-static 0.0.0.0 0.0.0.0 10.2.0.10

ip route-static 10.0.0.0 255.255.0.0 10.2.0.10

ip route-static 10.0.0.0 255.255.0.0 172.16.100.2

ip route-static 172.16.100.0 255.255.255.252 10.2.0.10

stelnet server enable

user-interface con 0

user-interface vty 0 4

authentication-mode aaa

return

- BR1-Agg-sw-1

sysname Agg-SW1

undo info-center enable

vlan batch 10 20 30 40 50 60 70 80 90 100

vlan batch 110 120 200 250

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

dhcp enable

dhcp snooping enable

diffserv domain default

drop-profile default

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password simple admin

local-user admin service-type http

interface Vlanif1

interface MEth0/0/1

interface Eth-Trunk1

description Core-sw1-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

dhcp snooping trusted

interface Eth-Trunk2

description Core-sw2-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

dhcp snooping trusted

interface GigabitEthernet0/0/1

port link-type access

port default vlan 80

interface GigabitEthernet0/0/2

port link-type access

port default vlan 90

interface GigabitEthernet0/0/3

port link-type access

port default vlan 70

interface GigabitEthernet0/0/4

interface GigabitEthernet0/0/5

interface GigabitEthernet0/0/6

interface GigabitEthernet0/0/7

interface GigabitEthernet0/0/8

interface GigabitEthernet0/0/9

interface GigabitEthernet0/0/10

interface GigabitEthernet0/0/11

interface GigabitEthernet0/0/12

interface GigabitEthernet0/0/13

interface GigabitEthernet0/0/14

interface GigabitEthernet0/0/15

interface GigabitEthernet0/0/16

interface GigabitEthernet0/0/17

interface GigabitEthernet0/0/18

interface GigabitEthernet0/0/19

interface GigabitEthernet0/0/20

interface GigabitEthernet0/0/21

eth-trunk 2

interface GigabitEthernet0/0/22

eth-trunk 2

interface GigabitEthernet0/0/23

eth-trunk 1

interface GigabitEthernet0/0/24

eth-trunk 1

interface NULL0

user-interface con 0

user-interface vty 0 4

port-group link-type

return

- BR1-Agg-sw-2

sysname Agg-SW2

undo info-center enable

vlan batch 10 20 30 40 50 60 70 80 90 100

vlan batch 110 120 200 250

stp mode rstp

cluster enable

ntdp enable

ndp enable

drop illegal-mac alarm

dhcp enable

dhcp snooping enable

diffserv domain default

drop-profile default

aaa

authentication-scheme default

authorization-scheme default

accounting-scheme default

domain default

domain default_admin

local-user admin password simple admin

local-user admin service-type http

interface Vlanif1

interface MEth0/0/1

interface Eth-Trunk1

description Core-sw1-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

stp edged-port disable

dhcp snooping trusted

interface Eth-Trunk2

description Core-sw2-UPLINK

port link-type trunk

port trunk allow-pass vlan 2 to 4094

dhcp snooping trusted

interface GigabitEthernet0/0/1

interface GigabitEthernet0/0/2

interface GigabitEthernet0/0/3

interface GigabitEthernet0/0/4

port link-type access

port default vlan 70

interface GigabitEthernet0/0/5

interface GigabitEthernet0/0/6

interface GigabitEthernet0/0/7

interface GigabitEthernet0/0/8

interface GigabitEthernet0/0/9

interface GigabitEthernet0/0/10

interface GigabitEthernet0/0/11

interface GigabitEthernet0/0/12

interface GigabitEthernet0/0/13

interface GigabitEthernet0/0/14

interface GigabitEthernet0/0/15

interface GigabitEthernet0/0/16

port link-type trunk

port trunk allow-pass vlan 100 120

interface GigabitEthernet0/0/17

description File-Storage-UPLINK

port link-type access

port default vlan 20

interface GigabitEthernet0/0/18

description NVR-UPLINK

port link-type access

port default vlan 90

interface GigabitEthernet0/0/19

description VOIP-UPLINK

port link-type access

port default vlan 80

interface GigabitEthernet0/0/20

interface GigabitEthernet0/0/21

eth-trunk 1

interface GigabitEthernet0/0/22

eth-trunk 1

interface GigabitEthernet0/0/23

eth-trunk 2

interface GigabitEthernet0/0/24

eth-trunk 2

interface NULL0

user-interface con 0

user-interface vty 0 4

return

- BR1-AC-1

sysname AC-Suez

undo http secure-server enable

set memory-usage threshold 0

ssl renegotiation-rate 1

vlan batch 100 120

authentication-profile name default_authen_profile

authentication-profile name dot1x_authen_profile

authentication-profile name mac_authen_profile

authentication-profile name portal_authen_profile

authentication-profile name macportal_authen_profile

dhcp enable

dhcp snooping enable

diffserv domain default

radius-server template default

pki realm default

rsa local-key-pair default

enrollment self-signed

ike proposal default

encryption-algorithm aes-256

dh group14

authentication-algorithm sha2-256

authentication-method pre-share

integrity-algorithm hmac-sha2-256

prf hmac-sha2-256

free-rule-template name default_free_rule

portal-access-profile name portal_access_profile

ip pool vlan120

gateway-list 10.2.12.1

network 10.2.12.0 mask 255.255.255.0

dns-list 8.8.8.8 8.8.4.4

aaa

authentication-scheme default

authentication-scheme radius

authentication-mode radius

authorization-scheme default

accounting-scheme default

domain default

authentication-scheme radius

radius-server default

domain default_admin

authentication-scheme default

local-user admin password irreversible-cipher $1a$7(vSY/0I*,$zcpkA=T\W6Vtum2'.y}O8bDuT].C&$*;,d8@I*5S$

local-user admin service-type http

interface Vlanif120

ip address 10.2.12.252 255.255.255.0

dhcp select global

interface Eth-Trunk1

description AGG-SW3-UPLINK

port link-type trunk

port trunk allow-pass vlan 100 120

dhcp snooping trusted

interface Eth-Trunk2

description AP-1-UPLINK

port link-type trunk

port trunk pvid vlan 120

port trunk allow-pass vlan 100 120

interface Eth-Trunk3

description AP-2-UPLINK

port link-type trunk

port trunk pvid vlan 120

port trunk allow-pass vlan 100 120

interface Eth-Trunk4

description AP-3-UPLINK

port link-type trunk

port trunk pvid vlan 120

port trunk allow-pass vlan 100 120

interface GigabitEthernet0/0/1

eth-trunk 4

interface GigabitEthernet0/0/2

eth-trunk 4

interface GigabitEthernet0/0/3

eth-trunk 3

interface GigabitEthernet0/0/4

eth-trunk 3

interface GigabitEthernet0/0/5

eth-trunk 2

interface GigabitEthernet0/0/6

eth-trunk 2

interface GigabitEthernet0/0/7

undo negotiation auto

duplex half

eth-trunk 1

interface GigabitEthernet0/0/8

undo negotiation auto

duplex half

eth-trunk 1

interface NULL0

snmp-agent local-engineid 800007DB03000000000000

snmp-agent

ssh server secure-algorithms cipher aes256_ctr aes128_ctr

ssh server key-exchange dh_group14_sha1

ssh client secure-algorithms cipher aes256_ctr aes128_ctr

ssh client secure-algorithms hmac sha2_256

ssh client key-exchange dh_group14_sha1

ip route-static 0.0.0.0 0.0.0.0 10.2.12.1

capwap source ip-address 10.2.12.252

user-interface con 0

user-interface vty 0 4

protocol inbound all

user-interface vty 16 20

protocol inbound all

wlan

traffic-profile name default

security-profile name default

security-profile name default-wds

security-profile name Corelink-Sec

security wpa-wpa2 psk pass-phrase %^%#;ov#(gw+_,h!FeM#1SSI(#DWGrV(o1Uhxm&K!yoT%^%# aes

security-profile name default-mesh

ssid-profile name default

ssid-profile name Corelink-SSID

ssid "Corelink Technologies"

vap-profile name default

vap-profile name Corelink-VAP

service-vlan vlan-id 100

ssid-profile Corelink-SSID

security-profile Corelink-Sec

wds-profile name default

mesh-handover-profile name default

mesh-profile name default

regulatory-domain-profile name default

air-scan-profile name default

rrm-profile name default

radio-2g-profile name default

radio-5g-profile name default

wids-spoof-profile name default

wids-profile name default

wireless-access-specification

ap-system-profile name default

port-link-profile name default

wired-port-profile name default

serial-profile name preset-enjoyor-toeap

ap-group name default

ap-group name CORELINK-APs

radio 0

vap-profile Corelink-VAP wlan 1

radio 1

vap-profile Corelink-VAP wlan 1

ap-id 1 type-id 69 ap-mac 00e0-fcfb-6980 ap-sn 21023544831074426025

ap-name Corelink-AP1

ap-group CORELINK-APs

ap-id 2 type-id 69 ap-mac 00e0-fc41-2440 ap-sn 2102354483109F269208

ap-name Corelink-AP2

ap-group CORELINK-APs

provision-ap

dot1x-access-profile name dot1x_access_profile

mac-access-profile name mac_access_profile

return