Core Link Technologies 
OSI Layer Classification & Purpose

Document Purpose

This document outlines the key protocols and technologies used in the "Core Link Technologies" topology. It classifies each item by OSI layer and operational purpose, providing a reference for design, simulation, and troubleshooting in enterprise networks.

1. OSI Layer-Based Classification

| Technology / Protocol | OSI Layer | Function |
| --- | --- | --- |
| WLAN (802.11) | Layer 1 / 2 | Wireless access medium |
| VLAN | Layer 2 | Logical segmentation of broadcast domains |
| STP / RSTP | Layer 2 | Loop prevention in switched networks |
| Eth-Trunk | Layer 2 | Link aggregation for redundancy and throughput |
| LLDP Enable | Layer 2 | Neighbor discovery and topology mapping |
| WPA / WPA2 | Layer 2 / 7 | Wireless encryption and authentication |
| Interface VLAN (SVI) | Layer 3 | IP interface for VLAN routing |
| Inter-VLAN Routing | Layer 3 | Routing between VLANs via Layer 3 device |
| IP Route Static | Layer 3 | Manual routing configuration |
| RIP v1 | Layer 3 | Dynamic routing protocol (distance-vector) |
| VRRP | Layer 3 | Gateway redundancy protocol |
| GRE Tunnel | Layer 3 | Encapsulation for VPN and routing over IP |
| CAPWAP | Layer 3 / 7 | AP control and provisioning |
| DHCP | Layer 7 | Dynamic IP address assignment |
| AAA | Layer 7 | Authentication, Authorization, Accounting |
| SSL | Layer 7 | Secure web encryption |
| SSH | Layer 7 | Secure CLI access |
| STelnet | Layer 7 | Secure Telnet (Huawei-specific) |
| FTP | Layer 7 | File transfer protocol |

2. Purpose-Based Grouping

A. Security & Access Control

• 	AAA – Centralized user authentication and accounting

• 	SSH / STelnet – Secure remote CLI access

• 	SSL – Encryption for web-based services

• 	WPA / WPA2 – Wireless encryption and user authentication

B. Routing & Redundancy

• 	IP Route Static – Manual route configuration

• 	RIP v1 – Basic dynamic routing

• 	VRRP – Virtual gateway failover

• 	GRE Tunnel – Encapsulation for secure routing over IP

C. Switching & Segmentation

• 	VLAN – Logical segmentation of Layer 2 domains

• 	Inter-VLAN Routing – Enables communication between VLANs

• 	Interface VLAN – Assigns IP to VLANs for routing

• 	STP / RSTP – Prevents Layer 2 loops

• 	Eth-Trunk – Aggregates multiple links

• 	LLDP Enable – Advertises device identity and capabilities

D. Wireless Infrastructure

• 	WLAN – Wireless LAN access

• 	CAPWAP – Centralized AP management and control

• 	WPA / WPA2 – Wireless security protocols

E. Services & Management

• 	DHCP – Automatic IP assignment

• 	FTP – File transfer between devices

• 	LLDP Enable – Device discovery and documentation

3. Simulation & Deployment Notes

• 	Huawei eNSP Compatibility: All protocols listed are supported in Huawei simulation environments. Use and commands for verification.

• 	Trunking & VLANs: Ensure interfaces are configured with and.

• 	VRRP: Use on both routers for redundancy.

• 	GRE: Tunnel interfaces require, and.

• 	CAPWAP: Requires AC-AP pairing and DHCP Option 43 for discovery.

4. Topology Use Case Summary

The "Core Link Technologies" topology is designed to:

• 	Simulate enterprise-grade Layer 2 and Layer 3 connectivity

• 	Demonstrate secure access and redundancy mechanisms

• 	Enable wireless integration and dynamic IP provisioning

• 	Provide a foundation for scalable, modular network design